Python kode til AfstandsSensor.py

import RPi.GPIO as GPIO

import time

from socket import *

BROADCAST_TO_PORT = 7005

s = socket(AF_INET, SOCK_DGRAM)

s.setsockopt(SOL_SOCKET, SO_BROADCAST, 1)

#GPIO Mode (BOARD / BCM)

GPIO.setmode(GPIO.BCM)

#set GPIO Pins

GPIO_TRIGGER = 18

GPIO_ECHO = 24

#set GPIO direction (IN / OUT)

GPIO.setup(GPIO_TRIGGER, GPIO.OUT)

GPIO.setup(GPIO_ECHO, GPIO.IN)

def distance():

# set Trigger to HIGH

GPIO.output(GPIO_TRIGGER, True)

# set Trigger after 0.01ms to LOW

time.sleep(0.00001)

GPIO.output(GPIO_TRIGGER, False)

StartTime = time.time()

StopTime = time.time()

# save StartTime

while GPIO.input(GPIO_ECHO) == 0:

StartTime = time.time()

# save time of arrival

while GPIO.input(GPIO_ECHO) == 1:

StopTime = time.time()

# time difference between start and arrival

TimeElapsed = StopTime - StartTime

# multiply with the sonic speed (34300 cm/s)

# and divide by 2, because there and back

distance = (TimeElapsed * 34300) / 2

return distance

if __name__ == '__main__':

try:

while True:

dist = distance()

data = str(dist)

print ("Measured Distance = %.1f cm" % dist)

s.sendto(bytes(data, "UTF-8"),('<broadcast>', BROADCAST_TO_PORT))

time.sleep(10)

# Reset by pressing CTRL + C

except KeyboardInterrupt:

print("Measurement stopped by User")

GPIO.cleanup()